EE-5214: Verification of Digital Systems

Lahore University of Management Sciences

Semester Project

AMBA AHB-Lite RAM Verification

Instructor: Dr. Tariq Ahmad

TAs: Sir Bilal Ahmed  |  Syed Ahsan Raza

Spring 2025

# Project Overview

This is a three-part project. You'll verify an open-source AHB-Lite RAM using simulation, SVA assertions, and formal methods. Each part builds on the last.

| Part | What you do | What you submit |
| --- | --- | --- |
| Part 1 — Simulation | Directed + random tests, scoreboard | Testbench, stimulus, scoreboard |
| Part 2 — Assertions | SVA checker bound to DUT | Checker module, bind file, write-up |
| Part 3 — Formal + Coverage | JasperGold FPV, covergroups | TCL script, assumptions, coverage report |

Do not touch the DUT RTL. Everything you write goes in separate files.

# Team Split  (3 students)

| Role | Who does what | Files |
| --- | --- | --- |
| A — Testing | Write the test plan. Implement directed tests for all HTRANS, HBURST, and HSIZE combinations. | testplan.md directed_tests.sv |
| B — Assertions & Formal | Write the SVA checker, bind it to the DUT, handle Part 3 formal run. | ahb_checker.sv bind_ahb.sv |
| C — TB & Coverage | Build the testbench top, constrained-random master, scoreboard, covergroups. | tb_top.sv scoreboard.sv ahb_cov.sv |

# Read Before You Start

Spec link (read sections 2, 3, and 4 — signals, transfers, bursts):

ARM IHI0033A — AHB-Lite Protocol Specification

Also review Lab 2 (interfaces), Lab 3 (constrained-random), and Lab 4 (coverage) notes from this course.

# DUT — What to Clone

Use the RoaLogic AHB3-Lite memory controller as your DUT. Clone both repos:

- Main DUT: github.com/RoaLogic/ahb3lite_memory

- Required package: github.com/RoaLogic/ahb3lite_pkg

- Optional — interconnect: github.com/RoaLogic/ahb3lite_interconnect

- Optional — APB bridge: github.com/RoaLogic/ahb3lite_apb_bridge

Get the DUT compiling with xrun before writing any testbench code.

# AHB-Lite Quick Reference

## Key signals

| Signal | Dir | What it does |
| --- | --- | --- |
| HCLK | In | Clock — everything happens on rising edge |
| HRESETn | In | Active-low reset |
| HADDR[31:0] | M→S | Address — must be aligned to HSIZE |
| HTRANS[1:0] | M→S | IDLE=00, BUSY=01, NONSEQ=10, SEQ=11 |
| HWRITE | M→S | 1=write, 0=read |
| HSIZE[2:0] | M→S | Transfer size: byte/halfword/word |
| HBURST[2:0] | M→S | SINGLE, INCR, WRAP4/8/16, INCR4/8/16 |
| HWDATA[31:0] | M→S | Write data — valid the cycle after address |
| HRDATA[31:0] | S→M | Read data — valid when HREADY=1 |
| HREADY | S→M | 0 = slave not ready, hold address phase |
| HRESP | S→M | 0=OK, 1=ERROR (must last 2 cycles) |

## Things to understand from the spec

- Address must be naturally aligned to HSIZE (word → HADDR[1:0]=00, halfword → HADDR[0]=0)

- NONSEQ starts a transfer; SEQ continues it — SEQ cannot follow IDLE

- Incrementing bursts must not cross a 1 KB boundary

- Wrapping bursts wrap at a power-of-2 boundary aligned to burst size × HSIZE

- HREADY=0 means hold everything — master must not advance

- HRESP=ERROR lasts exactly 2 cycles; first cycle HREADY=0, second HREADY=1

Part 1 — Simulation

## What to build

- Role A: Write testplan.md first — list every scenario you intend to test with expected signal values. Then implement directed tests for all HTRANS, HBURST, and HSIZE combinations. Include wait-state and boundary scenarios.

- Role C: Build the testbench top, a constrained-random AHB master class (constraints: alignment, 1 KB boundary, legal HTRANS state machine), and a write-then-read scoreboard.

## What to test (directed)

- SINGLE read and write for byte, halfword, and word sizes

- INCR4, INCR8, INCR16 — write then read back all beats

- WRAP4, WRAP8 — verify the address actually wraps at the right boundary

- HREADY=0 — slave inserts wait states, master must hold address phase

- Back-to-back transfers with no IDLE between

## Random testing

- Run at least 10,000 random transactions once directed tests pass

- Any scoreboard mismatch = potential DUT bug — log it

## Part 1 deliverables

| File | Who |
| --- | --- |
| testplan.md — scenario list, expected values per test | Role A |
| directed_tests.sv — all directed test tasks | Role A |
| tb_top.sv — top-level testbench | Role C |
| scoreboard.sv — write-then-read checker | Role C |
| ahb_cov.sv — initial covergroups (expanded in Part 3) | Role C |
| xrun_args.txt + covfile.txt | Role C |

Part 2 — Assertion-Based Verification

## What to build

Write an SVA checker module and attach it to the DUT using bind. The DUT source stays untouched.

bind ahb3lite_sram ahb_checker checker_i (.HCLK, .HRESETn, ...);

Inside the checker, set a default clocking and default disable iff at the top — saves you repeating it on every property.

## Protocol assertions to write

Read the spec. Figure out which rules apply to a simple RAM slave. Below are the minimum — add more if you find relevant rules.

- HADDR aligned to HSIZE on every NONSEQ and SEQ

- SEQ can only follow NONSEQ or SEQ — not IDLE or BUSY at burst start

- Fixed-length burst completes exactly the declared number of SEQ beats

- All address-phase signals held stable when HREADY=0

- HRESP=ERROR lasts exactly 2 cycles; HREADY=0 on cycle 1, HREADY=1 on cycle 2

- BUSY not legal inside a SINGLE transfer

- WRAP burst address stays within its naturally-aligned boundary

## Functional assertions to write

These check the RAM itself, not the protocol.

- Data written to address A must be readable from address A

- No memory location changes without a valid write transaction

- After reset, HREADY goes high and HRESP returns OK within a bounded number of cycles

- Byte write to address A does not modify bytes A+1, A+2, A+3

## A few things to keep in mind

- Name every assertion and add a comment with the spec section it comes from

- Write at least one cover property next to each assertion to confirm the scenario is reachable

- Deliberately inject violations and confirm each assertion fires

- Aim for quality over quantity — explain what each one checks and why it matters

## Part 2 deliverables

| File | Who |
| --- | --- |
| ahb_checker.sv — all assertions with names and comments | Role B |
| bind_ahb.sv — bind statement | Role B |
| assertion_report.md — table: name ¦ spec clause ¦ result | Role B |

Part 3 — Formal Verification & Coverage

## Task 3.1 — Formal proof (JasperGold)

Load the design into JasperGold FPV with your Part 2 checker attached. Run prove -task FPV.

- Write assume properties to constrain the tool to legal master behaviour — think about what the master can and cannot do per the spec

- For each assertion record the result: proven / counterexample / bounded

- If you get a counterexample, debug it — is it a real DUT bug or a missing assumption?

- Watch out for vacuity — if an assumption is too tight, assertions prove trivially; use cover properties to check

## Task 3.2 — Cover properties

Write cover property statements for interesting corner cases — at minimum:

- HREADY=0 held for 3+ consecutive cycles

- WRAP4 burst where the wrap actually happens

- Write immediately followed by read to the same address

- HRESP=ERROR generated by the slave

- Back-to-back NONSEQ with no IDLE

If a scenario turns out unreachable, explain why in your report.

## Task 3.3 — Functional coverage

Extend the covergroups from Part 1. Target above 90% on all coverpoints.

### Required coverpoints:

- All HTRANS values (IDLE, BUSY, NONSEQ, SEQ)

- All HBURST values

- All HSIZE values

- HWRITE (read and write)

- HREADY=0 observed at least once

- HRESP=ERROR observed

- Address space split into at least 4 bins

### Required crosses:

- HBURST x HWRITE

- HSIZE x HWRITE

- HTRANS x HREADY (especially SEQ with HREADY=0)

Run IMC, find uncovered bins, add targeted tests. Any bin you exclude needs a written reason.

## Part 3 deliverables

| File | Who |
| --- | --- |
| fpv_setup.tcl — analyze, elaborate, clock/reset, prove command | Role B |
| ahb_assumptions.sv — all assume properties (separate file) | Role B |
| ahb_cover.sv — cover property statements | Role B |
| ahb_cov.sv — updated covergroups from Part 1 | Role C |
| Part 3 report — results table, coverage screenshots | All |

# Final Report  (max 6 pages)

- Testbench architecture — draw a diagram showing DUT, master, scoreboard, checker

- Test plan summary — what you tested and why

- Assertion list — each property, spec clause, example waveform

- Coverage results — IMC screenshots before and after, any excluded bins explained

- Bugs found — description, which test or assertion caught it

- What surprised you or what you would do differently