# Formal Verification Project Report

AHB-Lite SRAM Slave (Role B)

Engineer: Abid Hussain

Role: Role B (Formal Assertions on Slave DUT) Target Protocol: AMBA 3 AHB-Lite Specification Verification Engine: Cadence JasperGold FPV Date: May 4, 2026

# Executive Summary

This document presents the comprehensive formal verification results for the AHB-Lite SRAM Slave interface. The primary objective of this verification phase (Role B) was to mathematically prove that the Slave design strictly adheres to the ARM AMBA 3 AHB-Lite protocol specification, while simultaneously verifying the functional integrity of the internal SRAM memory array.

Using Cadence JasperGold, properties were compiled and analyzed during the final regression run. The verification strategy successfully exposed critical hardware bugs in the RTL design while proving the baseline structural protocol compliance.

# Verification Methodology

## Formal Property Verification (FPV)

Unlike dynamic simulation, which relies on generating pseudo-random stimulus across millions of clock cycles, Formal Property Verification (FPV) exhaustively proves whether a property can ever be violated under any legal input condition. By utilizing SystemVerilog Assertions (SVA), the protocol rules were codified into mathematical properties.

## Symbolic Memory Tracking Architecture

Verifying memories formally typically results in state-space explosion because the tool attempts to track every bit of every address. To completely avoid this, a Symbolic Memory Tracking abstraction was utilized. The verification environment established a single, arbitrary symbolic address and tracked the two-phase AHB pipeline (Address Phase to Data Phase) exclusively for this address.

# RTL Defect Tracking (Counterexamples)

The formal tool identified critical protocol violations within the DUT. Trace analysis revealed the following flaws directly mapping to our assertion failures:

BUG 1: Invalid Address Phase Capture (p_transfer_only_when_ready)

Observation: The slave is ignoring the incoming HREADY signal and starting the data phase early. The protocol dictates that a valid address phase is only captured when

HREADY = 1. This early start creates a secondary failure regarding wait-state stability.

Actionable Fix: Gate the transfer capture condition in the RTL with the HREADY signal.

BUG 2: Wait State Protocol Violation (p_valid_wait_state_start)

Observation: The design asserts a wait state (pulling HREADYOUT low) without a preceding valid address phase. This is an illegal stalling of an idle bus, exacerbated by the failure to capture valid transfers properly.

Actionable Fix: Ensure HREADYOUT defaults to logic HIGH and is only pulled low when a valid NONSEQ or SEQ data phase is actively being processed.

# Merged Formal Assertions Verification Results

The table below unifies the specified assertions, categorized by type, alongside root causes and actionable fixes derived from the verification trace analysis.

| Assertion Name | Type | Spec Clause / Rule | Status | Root Cause / Observation | Actionable Fix / Notes |
| --- | --- | --- | --- | --- | --- |
| p_reset | Protocol | Reset (Sec 7.1.2): Slave outputsmust be OKAY | PASS | No issue. Reset behavior correct | None required |
| p_idle_busy_response | Protocol | Transfer Types (Sec 3.2): IDLE/ BUSY gets 0 wait-state | PASS | RAM behaves correctly | None required |

| p_no_x | Protocol | General: No unknown values on outputs | PASS | Outputs deterministic; No X propagation | None required |
| --- | --- | --- | --- | --- | --- |
| p_valid_wait_state_start | Protocol | Timing: Wait state must follow valid address phase | FAIL | Secondary failure due to early start / BUG FOUND | Fix p_transfer_only_when_ready first |
| p_two_cycle_error | Protocol | Error Response: ERROR must last exactly 2 cycles | PASS | No violation | None required |
| p_reset_recovery | Functional | Reset: System recovers after reset | PASS | System recovers; Stable behavior | None required |
| p_transfer_only_when_ready | Protocol | Basic Transfer: Transfer only when HREADY=1 | FAIL | DUT ignores HREADY and starts early | Gate with HREADY in transfer condition |
| p_read_stable_no_write | Functional | Memory: Memory stable without write | PASS | Correct RAM behavior | None required |

# Formal Assumptions (Master Constraints)

To accurately simulate an AMBA AHB-Lite Master and prevent the formal tool from injecting illegal inputs, strict environmental constraints were implemented. Below is the detailed tracking table

# Coverage and Reachability Analysis

To guarantee that the formal environment actively exercised complex DUT states (preventing vacuous assertion passes), 90 Cover properties were instrumented. The suite achieved an excellent 92.2% reachability hit rate, proving that the tool deeply explored sequences like back- to-back wait states, wrapping bursts, and specific byte-lane interactions.

# Conclusion and Next Steps

The verification suite successfully verified standard AMBA 3 protocol compliance for reset, X- propagation, and basic memory responses. However, the DUT is currently failing fundamental operational timing requirements.

Action Items for RTL Design (Role A):

- Update the transition logic from the Address Phase to the Data Phase to explicitly check that HREADY == 1.

- Ensure HREADYOUT defaults to logic HIGH and only stalls the bus legitimately.