# AMBA AHB-Lite RAM: Formal Verification Plan

Role B (Assertions & Formal Verification)

## Objective and Scope

This document outlines the Formal Verification strategy (Role B) for the AMBA AHB-Lite SRAM Slave. The primary objective is to build a robust formal correctness layer using SystemVerilog Assertions (SVA) and JasperGold Formal Property Verification (FPV).

#### Role B Responsibilities:

- Protocol checking against AHB-Lite specifications.

- Functional correctness verification of memory behavior.

- Formal verification environment setup (JasperGold).

- Coverage support for structural and functional completeness.

## Verification Strategy Overview

The verification architecture relies on an external, non-intrusive checker module utilizing three layers of verification:

- Layer 1: Protocol Assertions : Guarantees compliance with AMBA AHB-Lite bus rules.

- Layer 2: Functional Assertions : Validates proper SRAM read/write behavior and data integrity.

- Layer 3: Formal & Coverage Support : Implements constraints (assumptions) for legal master behavior and cover properties to prove state reach ability.

## Deliverables

| File | Description |
| --- | --- |
| ahb_checker.sv | Contains all protocol and functional SystemVerilog Assertions. |
| bind_ahb.sv | Binds the external checker module to the DUT. |
| ahb_assumptions.sv | Formal assumptions restricting input stimulus to legal AHB master behavior. |
| ahb_cover.sv | Cover properties to prove sequence reachability. |
| fpv_setup.tcl | JasperGold initialization and proof execution script. |
| assertion_report.md | Final results mapping assertions to specification clauses. |

## DUT Assumptions

The formal environment assumes the following baseline characteristics of the Design Under Test:

- The DUT is an AHB-Lite SRAM Slave.

- The design is fully synchronous to the positive edge of HCLK.

- Bus stalling is controlled exclusively via HREADYOUT.

- The slave supports two-cycle HRESP ERROR responses.

- There is no internal pipelining beyond the standard AHB Address/Data phase protocol.

## Protocol Verification Plan (AHB-Lite Rules)

### Address Alignment

All transfers must respect HSIZE boundaries during valid NONSEQ or SEQ phases.

| Transfer Size (HSIZE) | Required Alignment |
| --- | --- |
| Byte (8-bit) | Any address |
| Halfword (16-bit) | HADDR[0] == 0 |
| Word (32-bit) | HADDR[1:0] == 00 |

### Valid Burst Sequences

    - SEQ transfers must exclusively follow a NONSEQ or another SEQ transfer.

    - A burst must never initiate with a SEQ transfer.

    - SEQ cannot immediately follow an IDLE state.

### Burst Length Compliance

    - INCR4 bursts must contain exactly four sequential transfers.

    - WRAP bursts must complete the correct number of beats based on the burst type (4, 8, or 16).

### Wait State (HREADY) Hold Behavior

When the global HREADY signal is LOW (0), the master must hold all address and control signals stable.

    - HADDR remains stable.

    - HTRANS remains stable.

    - HWRITE remains stable.

    - HBURST remains stable.

### Two-Cycle ERROR Response

When the slave issues an ERROR, it must exactly follow the two-cycle AHB-Lite requirement:

    - Cycle 1: HRESP = 1 (ERROR), HREADYOUT = 0.

    - Cycle 2: HRESP = 1 (ERROR), HREADYOUT = 1.

### BUSY State Restrictions

    - BUSY transfers cannot be issued during a SINGLE transfer operation.

    - BUSY transfers are only valid within an active burst context.

### WRAP Boundary Rules

WRAP bursts must remain within their aligned memory boundaries and mathematically wrap at the correct address (Burst Size × HSIZE).

## Functional Verification Plan (RAM Correctness)

### Data Integrity (Write-to-Read)

Data written to address A must match the data returned when reading from address A, assuming no subsequent overwrites.

### Memory Stability

Memory contents must only change upon a valid write transfer. Spontaneous corruption due to HREADY

stalls or non-write phases (HWRITE = 0) is strictly prohibited.

### Reset Behavior

Following the assertion of HRESETn:

    - HREADYOUT must return to HIGH (1) within a bounded number of cycles.

    - HRESP must return to OKAY (0).

### Byte-Write Masking

During sub-word writes (byte or halfword), only the selected byte lanes within the memory array may change. All non-selected bytes must retain their previous values.

## Formal Constraints (Assumptions)

To prevent the formal engine from finding vacuous bugs, the checker must assume a legally behaving AHB-Lite Master.

#### Required Assumptions:

- Master Legality: HTRANS transitions strictly follow the AHB-Lite state machine.

- Burst Constraints: Bursts are properly initialized without undefined HBURST values.

- Address Ranges: The master does not generate addresses outside the allocated memory map.

- Reset Stability: The reset signal (HRESETn) remains stable until properly released asynchronously/synchronously.

## Formal Execution Plan (JasperGold)

### Phase 1: Setup and Elaboration

- Compile DUT and checker modules.

- Bind the checker to the DUT without RTL modifications (bind_ahb.sv).

### Phase 2: Constraint Application

- Apply clocking and reset definitions.

- Apply: ahb_assumptions.sv to restrict the mathematical state space to realistic master behaviors.

### Phase 3: Property Proving

Evaluate all assertions and classify them into the following formal states:

| Formal Result | Implication | Action Required |
| --- | --- | --- |
| Proven | Mathematically guaranteed correct. | None. Document in report. |
| Bounded | Proven up to N cycles. | Increase proof depth or investigate complexity. |
| CEX (Counterexample) | Bug found. | Debug waveform to identify DUT failure or missing constraint. |
| Vacuous | Proof passed, but trigger condition never occurs. | Fix over-constrained assumptions. |

### Phase 4: Coverage Validation

Ensure all cover properties hit. This proves that ERROR states, wait states, and burst wrap scenarios are mathematically reachable within the constrained environment.